Part 1: RISC-V Pipeline Implementation

Based on the selected subset of the RISC-V ISA, the datapath and pipeline control logic of the processor were implemented. Table 1 presents the instructions supported by this processor.

| R | Add,sub,xor,or,and, slt , sltu,sll |
| --- | --- |
| I | addi,andi,ori,xori,slti,sltui , slli |
| B | beq |
| J&I | Jal,jalr |
| I&S | sw,Lw |
|  | Lui , auipc |

Table 1 : instructions supported by this processor

Part 2: RISC-V Pipeline Implementation with Data Forwarding

To implement this version, the possible hazard scenarios in the processor were first identified. In this processor, data hazards and control hazards are present, while structural hazards are not considered. The data hazards in this processor are limited to Read After Write (RAW) hazards. WAR (Write After Read) and WAW (Write After Write) hazards do not occur.

A-Data Hazards

- Load-Use Hazard

Since the result of an LW instruction becomes available in the MEM (Memory) stage, a dependent instruction must be stalled until the required data becomes available. Therefore, a stall is introduced to ensure correct execution of the dependent instruction.

| LW X6 , 0(X5) | f | d | e | m | w |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addi x7 , x6 , 5 |  | f | d | d | e | m | w |  |  | Stall | m d |
| Beq x6,x0,len |  |  | f | f | d | e | m | w |  | wd |  |

- R-Type and I-Type Data Dependencies

This model includes only R-type and I-type instructions. The generated data must be made available to subsequent instructions, regardless of their instruction type.

The required data is provided through the forwarding units according to the detected data dependency. Therefore, no pipeline stall is required for these dependencies.

| Addi x5,x0 , 5 | f | d | e | m | w |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addi x6 ,x5 , 4 |  | f | d | e | m | w |  |  | f d |
| Add x7 , x6 ,x5 |  |  | f | d | e | m | w |  | ed md |
| BEQ x8 , x5 , BR |  |  |  | f | d | e | m | w | md wd |

B-Control Hazards

Control hazards are generated by instructions that change the program flow, such as branch and jump instructions. In this implementation of the RISC-V processor, these instructions are executed in the Decode (D) stage.

To correctly execute these instructions, the entry of a new instruction into the pipeline must be prevented when necessary. The control hazards considered in this implementation are caused by the BEQ, JAR, and JARL instructions.

- BEQ

For the BEQ instruction, the branch condition must first be evaluated. If the condition is satisfied, the branch is taken and a NOP instruction is inserted into the pipeline. If the condition is not satisfied, no branch occurs and the normal program flow continues.

| ed |  |  |  |  | w | m | e | d | f | Addi x7 ,x5 , 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | w | m | e | d | f |  | Beq x7 , x0 ,len |
| IF NOT TAKEN |  |  | w | m | e | d | f |  |  | Add |
| IF TAKEN |  | w | m | e | d | f |  |  |  | NOP |
|  | w | m | e | d | f |  |  |  |  | Len : |

- JAR and JARL

The JAR and JARL instructions are used for jump operations, including register-based jumps for return operations. To ensure correct execution, a NOP instruction must be inserted after the jump.

If a data dependency exists, forwarding must also be performed. Since the JAR instruction writes a value into a register before the following NOP instruction is inserted, the required data must be forwarded again from the MEM (M) and Write-Back (W) stages to the Decode (D) stage.

| JAR X5 , JUMP | f | d | e | m | w |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NOP |  | f | d | e | m | w | TO JUMP |  |  |  |
| JUPM : JAR X5 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | f | d | e | m | w | md |  |  |
| ALSO HERHE JAR X5 |  |  |  | f | d | e | m | w | w d |  |

C-Forwarding and Hazard Detection

To ensure correct program execution, data is forwarded from the EXE, MEM, and WR stages toward the Decode stage.

Two multiplexers are used in the datapath to select the appropriate forwarded data values. In addition, two other multiplexers are used to implement the Kill mechanism. Another multiplexer is used to insert a NOP instruction into the pipeline when a stall is required.

To detect hazards, the instruction opcode together with the Rs1D and Rs2D registers are considered.

The Decode unit receives the opcode signals and determines the corresponding ISA instruction type. This information then propagates through the pipeline, where data dependencies are detected by comparing the source registers with the corresponding RD registers in each pipeline stage.

This module is also responsible for generating the Kill signal associated with branch and jump instructions.

Figure 1 shows the datapath and controller of the pipeline hazard unit.

As shown in Figure 1, data is forwarded from the three pipeline stages EXE, MEM, and WR. This forwarding is performed by the F1 and F2 multiplexers, while their control signals are generated by the hazard control unit.

A STALL signal is also provided to freeze the pipeline when required, particularly in the case of LW instructions and load-use dependencies.

The figure also shows the generated KILL signal and the corresponding NOP values used to maintain correct pipeline execution.

Figure 1: shows the datapath and controller of the pipeline hazard unit.

Part 3: RISC-V Control Hazard Implementation

The hazard control unit is responsible for generating the control signals required for STALL, KILL, S_M1, and S_M2.

Figure 2 presents the complete structure of the hazard control unit.

The KILL signal is generated by the h_decode module, which determines the isa_type and generates the corresponding KILL signal.

The h_decode module receives op and zero as input signals. Based on the detected isa_type, the module determines whether a control hazard has occurred.

If the operation corresponds to JAR or JARL, the KILL signal is activated. For a BEQ instruction, if zero = 1, the branch condition is satisfied and the branch is taken; therefore, the KILL signal is activated.

Figure 2 : shows the implementation of the h_decode module.

The hazard_cont and encode_signal modules are responsible for detecting data dependencies and generating the required control signals.

The hazard_cont module compares the RD register of each pipeline stage with the Rs1D and Rs2D registers of the Decode stage. However, the ISA instruction type must also be known to correctly determine whether a dependency exists. This information is generated by the h_encode module.

When an LW instruction is present in the EXE stage and a dependent instruction requires its result, the STALL signal is activated.

The hazard_cont module generates the set1 and set2 signals. These signals are then connected to the encode_signal module to generate the corresponding S_M control signals.